Amlogic

Burning Tool Key Instruction

Revision: 0.5

Release Date: 2024-01-25

Copyright

© 2024 Amlogic. All rights reserved. No part of this document may be reproduced, transmitted, transcribed, or translated into any language in any form or by any means without the written permission of Amlogic.

Trademarks

, and other Amlogic icons are trademarks of Amlogic companies. All other trademarks and registered trademarks are property of their respective holders.

Disclaimer

Amlogic may make improvements and/or changes in this document or in the product described in this document at any time and without notice.

This product is not intended for use in medical, life saving, or life sustaining applications, nor in applications where failure or malfunction of an Amlogic product can reasonably be expected to result in personal injury, death or severe property or environmental damage.

Circuit diagrams and other information relating to products of Amlogic are included as a means of illustrating typical applications. Consequently, complete information sufficient for production design is not necessarily given. Though every effort has been made to ensure that this document is current and accurate, Amlogic makes no representations or warranties with respect to the accuracy or completeness of the contents presented in this document.

Amlogic products may be subject to unidentified vulnerabilities or may support established security standards or specifications with known limitations. Amlogic accepts no liability for any vulnerability.

Use of the materials may require a license of intellectual property from a third party or from Amlogic. This document conveys no express or implied licenses to any intellectual property rights belonging to Amlogic or any other party. Any links to third-party websites included in this document are for your convenience only. Amlogic does not endorse and is not responsible for such websites and their practices, including privacy practices, availability, and content.

Amlogic shall in no event be liable for any claims relating to or arising out of this document or any use or inability to use hereof.

Contact Information

Website: www.amlogic.com

Pre-sales consultation: contact@amlogic.com

Technical support: support@amlogic.com

Revision History

Issue 0.5 (2024-01-25)

This is the 0.5 release.

Added key format and merged into other documents.

Issue 0.4 (2023-10-09)

This is the 0.4 release.

Added eFuse instruction.

Issue 0.3 (2020-09-15)

This is the 0.3 release.

V3 tool key burning usage guide initial version.

Issue 0.2 (2020-06-11)

This is the 0.2 release.

Compared to last version, added error load information.

Issue 0.1 (2019-03-26)

This is the initial release.

Contents

Revision History	ii

Contents	iii

1.	Overview	1

2.	Modifying Burn Image and Burn Key Option	1

2.1	Tool Main Interface	1

2.2	Selecting the Burning Package Level	1

2.3	Key Configuration Interface	2

3.	Copy the Key File to the Tool License Directory	4

4.	Modifying the Configuration file KeysProviderCfg.ini	5

5.	Key Format Configuration	6

5.1	Configuration of Used/uuid/sn/deviceid	6

5.2	Configuration of Mac/BlueTooth/Wifi	8

5.3	Configuration of HDCP	8

5.4	Configuration of Widevinekeybox	9

5.5	Configuration of HDCP2/HDCP2_RX	9

5.6	Configuration of HDCP14_RX	10

5.7	Configuration of HDCP22_fw_private	10

5.8	Configuration of rxhdcp20	10

5.9	Configuration of aml_hdcp_key2.2	11

5.10	Configuration of dtcp	11

5.11	Configuration of Secure_boot_set	12

5.12	Configuration of Secure_boot_ext	12

5.13	Configuration of Playreadykeybox	12

5.14	Configuration of Playreadykeybox3.0	12

5.15	Configuration of Telecomkeybox/usidonly/rsa_puk_e/rsa_puk_n/aes.key/bootstrap.key/amlogic_set	12

5.16	Configuration of hwid	12

5.17	Configuration of SSL	13

5.18	Configuration of acs	13

5.19	Configuration of Attestationkeybox	13

5.20	Configuration of Netflix_mgkid	14

5.21	Configuration of Dir Folder Format Key	14

5.22	Bind Burning of ATTEST DEV ID and USID	14

6.	Burning Key image as common image	16

7.	Burning of eFuse	18

7.1	Pattern Mode	18

7.2	Obj Mode	18

7.3	Example of Burning Configuration	18

Overview

This document is used to explain how to use the V3 burning tool to burn keys. Using a burning tool to burn a Key includes the following four steps:

Modify burn image, add burn key option with CustomizationTool.

Add a key name list file to the burning package, usually using the key function of Customization Tool to modify the burning package and add the option to burn the key.

Check key configuration ini (normally install in C:\Amlogic\Aml_Burn_Tool\V3\KeysProviderCfg.ini) and KeysProviderCfg.ini is used for parse license file to meet the demand.

Burn key like a regular burning package.

Modifying Burn Image and Burn Key Option

First, add key information to the burned image and notify the burning tool which keys need to be burned.

Tool Main Interface

After installing the customized tool, click on CustomizationToolexe. Use customized tools to add options for burning keys (Figure 2.1).

Figure 2.1 Customization Tool

Selecting the Burning Package Level

Click "Load" on the main interface, and a decompression options dialog box will pop up (Figure 2.2). Import the original image compression package and select "Key Settings (Set Key Name for Burning)".

Figure 2.2 choose “Setting Key” options

Key Configuration Interface

There are some key options to be chosen in the Key configuration interface.

Figure 2.3 Set the configuration file of the keys to be burn

List box has three columns.

- The first column "Key name" is the name of the Key to be burned (the name specified by the Key during burning is consistent with the Key name of the DTS during the compilation of the burning package). The existing Key name on the Key page only displays its function, indicating that it is a Key name supported by the public version. If there is no required Key name on the Key page, it can be added through the keyboard.

- The second column "File Name" is the Key file name corresponding to the Key name, which is the name of the Key file to be placed in the License directory during burning (this column is only for prompt purposes and may not be filled in);

- The third column "Burn or not" indicates the selected status of the key, and checking it indicates that the key needs to be burned; If there is only the first column name but the third column is not checked, it will not be burned.

Key storage media include unifykey, provisionKey, and eFuse.

- The key name of Unifykey must be configured in the/Unifykey of dts.

- The Key name of the Provision needs to start with KEY-PROVISION_, such as KEY-PROVISION-MKGID.

- The eFuse Key name includes two different formats: pattern mode and obj mode, namely binary mode and text mode. Old chips are usually in pattern mode, while new chips are usually in obj mode.

The pattern mode is in binary format, and for single or multiple secure boot eFuse, the Key name is secure_boot_set. The key name for other non-secure boot is amlogic.pattern_xxx, where there can be multiple patterns, such as amlogic.pattern_drm and amlogic.pattern_usd_pwd.

The pattern of secure boot, also known as secure boot set, once it enters the burning state, after a power outage and restart, the SoC enters the encrypted state. When using tools to burn either secure boot set or amlogic_pattern_xxx, it is not processed. For eFuse patterns that have already been burned with secure boot enabled, if you need to burn other patterns, you can use methods other than USB tools to burn them. It is recommended that the factory only burn one or more eFuse at one workstation during the production process.

The new version of the chip can use obj mode, which is text format. The key name burned by a single secure boot eFuse is secure_boot_objects, while the key name for multiple non secure boot eFuse is amlogic_euse obj_xxxx. In development mode, for eFuse obj that have already been burned with secure boot enable, other eFuse obj can be burned separately, but can only be burned once.

The user can select the related key names, check the checkbox, and click SetKeycfg button (Figure 1.3). The tool will set the keys to be burnt into keys.conf file. This file is needed when re-generating the image package. If the required key is not listed in the interface, the user can double click the items in the first column and edit the key name. notice this name need the same as dts key name when build.

Copy the Key File to the Tool License Directory

Place the Key file to be burned into the license directory of the tool installation directory.

The path to the License directory needs to be checked in keysProvidercfg.ini. If the license directory does not exist, it can be manually created.

The [licensePath] in the configuration file KeysProviderCfg.ini under the burning tool indicates the location of the Key directory.

The default license path can be modified.

For specific files, it is necessary to find them after the Key name specified in KeysProviderCfg.ini. Usually, each key is represented by * *=* * *, with the left side of the "=" sign indicating the name of the key (consistent with the name of the dts when compiling the image) and the right side indicating the name of the key file. We need to put the right key file in the corresponding license path.

Example: [MacManager]

Mac=mac_ether.ini

Display the Mac's burn key file as mac_ether.ini and put it in the directory specified by [licensePath].

[OnlyOneManager]

Secure_boot_set=SecureBOOT.SET

The key name of the display tool is secure_boot_set, and the key file is SECUREBOOTSET in the license directory.

Modifying the Configuration file KeysProviderCfg.ini

The configuration file for Key, KeysProviderCfg.ni, is in the installation directory of the burning tool and configured according to the actual situation.

Based on the key name selected in the Customization tool, refer to this ini to determine how to parse a specific Key name.

As shown in the configuration above, when the customization is configured with Key name KEY-PROVISION-WIDEVINE, the tool uses this Key name to search for ini. The file format for this Key is [KeyDirManager], which is a folder format. Each file under the widevinekeybox.dir folder under the license is a Key value file.

Each board typically uses this directory method to configure the provision key for burning different keys. After importing each type of key, the tool will generate an ini configuration file. When importing a new key folder, the old configuration. ini file needs to be deleted.

The value to the left of "=" in ini represents the Key name and cannot have two identical Key names.

Key Format Configuration

Configuration of Used/uuid/sn/deviceid

Usid.ini is the key file for usid, uuid.ini is the key file for uid, sn.ini is the key file for sn, and deviceid.ini is the key file for deviceid. These key files need to be placed in the license directory of the tool directory during burning.

Its format is as follows:

[Group1]

usid = ShiningStar<1>MBX<2>

param_1_format = %04x

param_1_start = 0000

param_1_end = FFFF

param_1_used = 0x0

param_1_total = 1000

param_2_format = %04x

param_2_start = 0000

param_2_end = 0FFF

param_2_used = 0x0

param_2_total = 1000

[Size]

Size = 22

[fragment]

fragment =

[Group *] represents a set of keys, and users can use [Group1], [Group2]. Each group [Group *] can use a different format.

Usid=ShiningStar<1>MBX<2>is a string of usid, uuid=Amlogic<1>MBX<2>is a string of uuid, sn=Amlogic<1>MBX<2>is a string of sn, deviceid=Amlogic<1>MBX<2>is a string of deviceid. The string of a Key is usually divided into variable and fixed parts, with the fixed part being the given string, such as ShiningStar and MBX; The variable part is a fixed length variable string, configured as<1>, <2>.

For the variable part, param_ * _format specifies the format and length, which are described in several situations:

% 04x or% 04X: represents a string length of 4, which is a hexadecimal number.

% 04d or% 04D: represents that the length of the string in this section is 4, and the string is a decimal number.

Take the variable part key in the numerical form of param_ * _start+param_ * _used each time and fill in any missing digits with 0.

If R is added to the param_ * _format, it represents that the variable key part is a random number, which is selected between param_ * _start and param_ * _end.

- % 04xR or% 04XR: represents a string length of 4, which is a hexadecimal random number.

- % 04dR or% 04DR: represents that the length of the string in this section is 4, and the string is a decimal random number.

Param_ * _start represents the starting number of variable keys, param_ * _end represents the end of numeric characters in the key, and param_ * _used represents recently used keys, which will be automatically set during the use process.

Param_ * _total represents the total number of variable keys. If using a random number, such as% 04xR, param_ * _total must be set.

If the configuration file has multiple variable parts: <1>, <2>, The value of the tool will first be selected from <1>, and when the value of <1> reaches the set end, it will be reset to zero. The param_2_used number of <2> will be added by one.

Size=represents the length of the key string and must be set.

Fragment=is the key that the tool recycles during use. If a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If the user wants to burn a key string of 9999 keys from Amlogic0001M200 to Amlogic9999M200 using the USID, the key file should be set as follows:

[Group1]

usid = Amlogic<1>M200

param_1_format = %04d

param_1_start = 0000

param_1_end = 9999

[Size]

Size = 15

The keys burned in this way are Amlogic0001M200, Amlogic0002M200... and so on, until Amlogic9999M200.

Configuration of Mac/BlueTooth/Wifi

Mac_ether.ini is the configuration file for Mac, mac_bt.ini is the configuration file for Bluetooth, and mac_wifi. ini is the configuration file for WiFi. These keys are placed as license files for each type in the license folder under the tool directory. They have the same file format, as shown below:

[Group1]

Start=00:0f: a3:45:9b: 12

End=00:0f: a3:45: a1:34

Total=1570

Used=2

Current=0f: 0f: a3:45:9b: 14

[Group *] represents a set of keys, which users can use [Group1], [Group2], and each set of keys uses a different format.

Start=represents the starting position of the MAC; End=represents the final position of the MAC, which must be configured.

Total represents the total number of Macs, which does not require configuration and will be automatically calculated by the tool.

Used represents the number of macs used, and defaults to 0 when not set.

Current represents the currently used Mac, which can be automatically generated by the tool without setting it.

Configuration of HDCP

The key file name of HDCP is HDCP-LIENCE, with 308 bytes per key. The key file has 4-byte headers, with a total length of 4+N * 308, where N represents the number of keys. Place HDCP-LIENCE in the license folder under the tool directory.

HDCP-LIENCE.ini is the configuration file for hdcp. The first use of the tool will generate the HDCP-LIENCE.ini file, which does not require user settings.

The format of the Ini file is as follows:

[general]

Current=2

[fragment]

Fragment=0

Among them, [general] current represents the sequence number of the key to be used, which will continue to increase during use.

Fragment=is the key that the tool recycles during use. If a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the HDCP-LIENCE.ini file does not need to be set, and the tool will generate it.

Configuration of Widevinekeybox

The key file name of Widelinekeybox is Widelinekeybox.bin, with 148 bytes per key. The key file has 4-byte headers, with a total length of 4+N * 148, where N represents the number of keys. Put the file in the license folder under the tool directory.

Widevinekeybox.ni is the configuration file for widevinekeybox. The first time the tool is used, it will generate a widevinekeybox.ni file, which does not require user settings.

The format of the Ini file is as follows:

[general]

Current=2

[fragment]

Fragment=0

Among them, [general] current represents the sequence number of the key to be used, which will continue to increase during use.

Fragment=is the key that the tool recycles during use. If a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the widevinekeybox.ini file can be generated by the tool without setting it.

Configuration of HDCP2/HDCP2_RX

The key file name for HDCP2 is HDCP2-LIENCE, and the key file name for HDCP2RX is HDCP2RX-LIENCE, with 902 bytes per key. This type of Key file has 40-byte Key headers and N Key data, with a total length of 40+N * (key_size-40). The HDCP2-LIENCE and HDCP2RX-LIENCE files are in the license folder of the tool directory.

HDCP2_LIENCE.ini is the configuration file for hdcp 2, and HDCP2RX-LIENCE.ini is the configuration file for hdcp2RX. The first use of the tool will generate HDCP2_LIENCE.ini and HDCP2RX-LIENCE.ini files, which do not require user settings.

The format of the Ini file is as follows:

[general]

Current=2

[fragment]

Fragment=0

Among them, [general] current represents the sequence number of the key to be used, which will continue to increase during use.

Fragment=is a key that is recycled during use. When a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use during burning, without the need for user configuration. The tool will automatically process it.

If you need to use it from the first one, the HDCP2-LIENCE.ini and HDCP2RX-LIENCE.ini files can be generated by the tool without setting them.

Configuration of HDCP14_RX

The key file name for HDCP14-RX is aml_hdcp_key1.4. bin, with each key length of 348 and a total length of N * key_size. aml_hdcp_key1.4. binini is the configuration file for hdcp14_rx, located in the license folder of the tool directory.

The format of aml_hdcp_key1.4. bin. ini is as follows:

[general]

Current=0

[fragment]

Fragment=

[general] current represents the sequence number of the key to be used, which will continue to increase during use.

Fragment=is the key that the tool recycles during use. When a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the aml_hdcp_key1.4. bin. ini file can be generated by the tool without setting it.

Configuration of HDCP22_fw_private

The key file name for hdcp22_fw_private is hdcp22_fw_private.bin, where the key is the entire file content. The M8 chip is 768 bytes, and chips 905 and later are 32 bytes. They are placed in the license folder in the tool directory. No configuration files are generated during use.

Configuration of rxhdcp20

The key file name for rXHDCP20 is RXHDCP20-LIENCE, with each key length of 368 byte and 4 byte headers. The total length is 4+N * 368, where N is the number of keys. RXHDCP20-LIENCE.ini is the configuration file for rXHDCP20 and is placed in the license folder in the tool directory.

The format of rxhdcp20-LIENCE.ini is as follows:

[general]

Current=0

[fragment]

Fragment=

[general] Current represents the sequence number of the key to be used, which continuously increases during use,

Fragment=is the key that the tool recycles during use. When a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the RXHDCP20-LIENCE.ini file can be generated by the tool without setting it.

Configuration of aml_hdcp_key2.2

The key file name for aml_hdcp_key2.2 is aml_hdcp_key2.2. bin, with each key containing 2248 bytes and a total length of N * 2248. N is the number of keys. Put all the files in the license folder under the tool directory.

Aml_hdcp_key2.2. bin. ini is the configuration file for aml_hdcp_key2.2. The first use of the tool will generate the aml_hdcp_key2.2. bin. ini file, which does not require user settings.

The format of the Ini file is as follows:

[general]

Current=2

[fragment]

Fragment=0

[general] Current represents the sequence number of the key to be used, which continuously increases during use,

Fragment=is the key that the tool recycles during use. When a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the aml_hdcp_key2.2. bin. ini file can be generated by the tool without setting it.

Configuration of dtcp

The key file name of dtcp is dtcp.bin, with 584 bytes per key. The key file has 4 byte headers, and the total length is 4+N * 584. N is the number of keys. Put all the files in the license folder under the tool directory.

Dtcp.bin.ini is the configuration file for dtcp. The first use of the tool will generate a dtcp.bin.ini file, which does not require user settings.

The format of the Ini file is as follows:

[general]

Current=2

[fragment]

Fragment=0

[general] Current represents the sequence number of the key to be used, which continuously increases during use,

Fragment=is the key that the tool recycles during use. When a key is obtained but not successfully burned, the tool will recycle the failed key as a fragment for future use. No user configuration is required, and the tool will automatically process it.

If you need to use it from the first one, the dtcp.bin.ini file can be generated by the tool without setting it.

Configuration of Secure_boot_set

The key file name for secure_boot_set is SECUREBOOTSET, where the key is the entire file content, and the length is defined by the developer and placed in the license folder in the tool directory. No configuration files are generated during use.

Configuration of Secure_boot_ext

The key file name for secure_boot_ext is SECUREBOOTSET.bin, where the key is the entire file content, and the length is defined by the developer and placed in the license folder in the tool directory. No configuration files are generated during use.

Configuration of Playreadykeybox

The Key of Playreadykebox is generated during use, and during burning, it is necessary to add a certificate file and a burning library for Playreadykebox. The certificate files are bgroupcert. dat and zgpriv. dat, where genDevCert. dll is placed in the tool directory and the certificate file is placed in the license folder under the tool directory.

Configuration of Playreadykeybox3.0

The key names for Playreadykeybox 3.0 are prpubkeybox and prprivkeybox, and the corresponding key files are certificates bgroupcert. dat and zgpriv protected. dat. The key is the entire file content, and the length is defined by the developer. These two certificates are used as keys and placed in the license folder in the tool directory. These two types of keys will not generate configuration files.

Configuration of Telecomkeybox/usidonly/rsa_puk_e/rsa_puk_n/aes.key/bootstrap.key/amlogic_set

For these keys with special names, the Key file is their file name, with a length equal to the entire file content, and is placed in the license folder under the tool directory. No configuration files are generated during use.

Configuration of hwid

Hwid. ini is the key file of Hwid, which should be placed in the license directory of the tool directory during burning.

The configuration file for this type of Key is unique, with a specific combination of fixed domain bytes and non-fixed domain bytes, such as:

ODM=1

RSV=0

Start=20150144

End=20150317

Used=2.

ODM and RSV are fixed field bytes, and tools combine these two fields into fixed characters based on a certain combination. Start and End are active fields, and variable bytes are obtained within the range of Start~End. They are combined with the previous fixed characters to form a complete Key according to a certain algorithm.

Configuration of SSL

SSL. ini is the key file for SSL, which should be placed in the license directory of the tool directory during burning. The number of SSL keys is determined by the first four bytes, and the length of each key is determined by the first two bits of the key data.

Configuration of acs

ACS_key.txt is the key file of ACS, which should be placed in the license directory of the tool directory during burning. The Key configuration file acs_key.txt.ini has the following format:

[ACS]

Total=19

Current=0

[fragment]

Fragment=

Total is the total number of ACS, and current represents the current key used. Each line of this key is a key, and during burning, a specific line of the key file is read based on the current as the current key.

If you need to use it from the first one, acs_key.txt.ini will be automatically generated when the tool is loaded and does not require user configuration.

Configuration of Attestationkeybox

The key of Attention keybox usually exists in the form of a suffix. keybox and is placed in the unifykey folder. The unifykey folder must be placed in the license directory of the tool directory during burning. After the tool loads and burns the img for attestationkeybox, it will generate the attestationkeybox.ini configuration file, which tells the tool about the number of keys, usage status, and other information. If the above approach is ensured to be correct, and the key obtained by the tool after loading is still 0, it may be due to the retention of the original attestationkeybox.ni. Delete the attestationkeybox.ni configuration and reload img. The format of the attestationkeybox.ni is as follows:

[KeyDir]

ONEDAY-MP_00000 1. keybox=0

ONEDAY-MP_00000 2. keybox=0

[Size]

Size=8951

[fragment]

fragment=

Configuration of Netflix_mgkid

The key of netflix_mgkid is usually NFANDROID CSV exists in the form and is placed in the license directory of the tool directory.

In the following form:

Each row has a Key.

After loading and burning the img of netflix_mgkid, the tool will automatically generate the NFANDROID.csv.ini configuration file, which does not require configuration.

[NFAND]

Total=10

Current=0

[Size]

Size=135

[fragment]

fragment=

Configuration of Dir Folder Format Key

A key in the form of a folder, where all key files are placed in one folder, such as: attestationkeybox=unifykey, where the left side is the key name, and the right side is the key folder name. Applicable to the configuration of this type, the tool will automatically generate an. ini configuration file named after the file name after loading the img of this type, which is used to record the usage of the file Key and does not require configuration. Note that if you want to replace the Key file, you need to delete the ini file and do not place files that are not keys in this folder.

[KeyDirManager]

Attestationkeybox=unifykey

Aml_hdcp_key2.2. ta=aml_hdcp_key2.2. dir

Bind Burning of ATTEST DEV ID and USID

On Android 12 and above platforms, there is a special method for generating encryption for ATTEST-DEV-ID0-BOX, and CTS testing requires burning ATTEST-DEV-ID0-BOX and usid.

The Attest-Dev-ID_Box and usid need to be bound and burned to the device one by one. The Attest-Dev-ID-Box file can be used as both the content of the Attest-Dev-ID-Box and the content of the usid. The file name (without the suffix) can be used as the usid, and the content of the key file can be used as the Attest-Dev-ID-Box.

Name the folder containing the encrypted keys for AttestDefIDKeys and place it in the [CommonlicensePath] directory set by KeysProviderCfg.ni. Do not include the usid.ini and KEY-PROVISION-ATTEST-DEV-ID-BOX. ini files in this directory for the first time, as the tool will use them to cause interference when importing these keys for the first time. Without these files, the tool will automatically generate them.

Configure the key files corresponding to these two types of keys in the [Customization] section of KeysProviderCfg. ini, where both are AttestIDKeys.

[Customization]

usid=AttestIDKeys

KEY_PROVISION_ATTEST_DEV_ID_BOX=AttestIDKeys

After the burning tool loads the bound burning package, it will generate two ini configuration files: usid.ini and KEY-PROVISION-ATTEST-DEV-ID-BOX. ini, which respectively record the configuration and usage of these two keys. When burning, it will burn the file name prefix and ke file content of each key to achieve one-to-one binding. For example, if the content of the burned usid is the prefix "Aml_001" in the Aml_001.xml.factory.inc file, then the content of the burned KEY-PROVISION-ATTEST-DEV-ID-BOX is the binary data of the Aml_001.xml.factory.inc file.

Burning Key image as common image

When all the steps are ready, select and load the key image file, then it begins to burn image. Users consider correct configuration when list show key name and amount.

Figure 4.1 Key Config Right

If the key configuration is incorrect, the burning tool will search for the error type based on the interface return value and pop up the corresponding error message (Figure 6.2). The information corresponding to the error type can be found in ErrorKeyproviderMsg.xml (Figure 6.3).

Figure 6.2 Key error message

Figure 6.3 Error type corresponding information

According to the error prompt, follow the second and third steps to check the Key configuration.

Figure 6.4 Set overwrite key

Before burning, you can choose whether to overwrite the burning key (Figure 6.4). That is, when the burning device already has a key, this option can specify whether to overwrite the old key with the new license key. If not selected, the old key will not be overwritten, and the device still has the existing key.

Burning of eFuse

eFuse burning is inject Efuse Key to chip eFuse，such as secure boot eFuse, audio_id.efuse, dgpk. efuse etc.

eFuse have pattern mode and obj mode, old chip usual is pattern mode, new chip usual is obj mode.

Pattern Mode

Normal old chip is pattern mode, that is Hex mode.

The keyname is “secure_boot_set” when single secure boot eFuse.

When have multiple efuse, named “secure_boot_set” when burn secure boot eFuse, another no_secureboot key named “amlogic_pattern_xxx”, and no_secureboot efuse pattern can be multiple.

Secure boot pattern is secure_boot_set, once burned, after power off and restart, SoC enters the encrypted state, any attempt to use the tool to burn secure_boot_set or amlogic_pattern_xxx will be ignored and no longer processed.

The efuse pattern of secure boot enable has been burned, and other patterns need to be burned. You can use other methods other than usb tools to burn. We emphasize and recommend that the factory production process only burn one or more efuses in one station.

Obj Mode

Normal New chip is obj mode, that is Txt mode.

The Single secure boot efuse, keyname is “secure_boot_objects”, multiple no_secureboot efuse keyname is “amlogic_efuse_obj_xxxx”.

In Devloper mode, other eFuse objects also can burn when device have burn secure boot enable.

Example of Burning Configuration

When have SECURE_BOOT_SET, and audio.efuse, the audio.efuse can burn to all devices, then in KeysProviderCfg.ini, [OnlyOneManager] config as secure_boot_set=SECURE_BOOT_SET, keyname is secure_boot_set, the “=” left, file name is SECURE_BOOT_SET, “=” right.  Otherwis, other efuse config as amlogic_efuse_obj_audio = audio.efuse, keyname is amlogic_efuse_obj_audio, the “=” left, file name is audio.efuse, “=” right.

as：

[OnlyOneManager]

secure_boot_set=SECURE_BOOT_SET

amlogic_efuse_obj_audio = audio.efuse

When different efuse files burn to different devices, the folder configration be needed, such as dgpk2, config in [KeyDirManager] amlogic_efuse_obj_dgpk2=dgpk2Dir, keyname is amlogic_efuse_obj_dgpk2, the “=” left, folder name is dgpk2Dir, “=” right, dgpk2 efuse files is in the folder.

as：

[KeyDirManager]

amlogic_efuse_obj_dgpk2=dgpk2Dir